Colour - a simplified view!

Basic Colour Science

Colour Science, of which Colour Management is a part, is an important part of a photographer’s toolbox. However, as a subject in its own right, Colour Science is a Masters/Phd level subject! Colour science has its spiritual home at the Rochester Institute of Technology (RIT) located in Rochester New York state. This, coincidently, is also the home of Kodak.

I, along with probably a lot of photographers, am by no means an expert in colour science nor in the art of colour management, but I do have a vast amount of experience, most of which was learned the hard way when I was responsible for converting RAF Photography from conventional ‘wet film’ to digital! All of this done in the 90’s when little was understood about the subject.

Light is clearly crucial to photography and there is therefore a need to understand the very basics of colour science in order to get the best from it. So, lets begin with light.

Visibly transmitted light forms a small part of the Electro-Magnet (EM) Spectrum, from a wave length of 380nm (Blue) to 720nm (Red). It is normally described as having 7 main colours Violet, Indigo, Blue, Green, Yellow, Orange and Red (the colours of the rainbow). In practice the colours transition through from Violet to Red in infinitely small amounts. When mixed together the whole of this spectrum of light forms white light.

The human eye ‘sees’ light through a series of rods and cones. The cones, concentrated in the centre of the retina essentially perceive colour in the RGB channels. In addition to colour, in bright situations they also provide our visual acuity (detail). The eye is capable of discerning up to around 8 to 10 million colours. The rods provide visibility in poor light allowing us to see peripheral movement and low light sight. The eye has a normal latitude of around 10 – 12 stops but can adjust, over time, to around 30 stops!

This is a graphical representation of the response of the eye to colour. It shows that the eye’s colour response is concentrated in the green/yellow part of the spectrum. Up to 1 in 12 males can have red/green colour blindness due to a loss or reduction in the green/red cones. This does not affect females to the same extent (1in 200).

However, it doesn’t stop here! Signals are sent via the optic nerve to the brain which is probably the most powerful parallel processor known. Although the eye only ‘sees’ about 10 million colours, the brain is able to ‘perceive’ many, many millions of colours.

Camera sensors also ‘sense’ the EM spectrum and together with the Bayer colour filter have the response shown in the graph (note that this extends from around 350nm up to 1050nm,into the Near Infra Red). Near Infra Red wavelengths are prevented from reaching the sensor by an IR cut filter. A 14 bit sensor can record more distinct colours that the human eye alone but the eye/brain combination can perceive many more.

In 1931 the Commission Internationale de l’Eclairage (CIE) or International Commission on Illumination, produced a 3-dimensional colour space to describe the colours perceived by human vision. To make this colour space more easily viewable it often seen reduced to a 2-dimensional representation.

In imaging applications the visible spectrum is further reduced to 3 main components; Red, Green and Blue (RGB). Again, if these are mixed the result is white light. This is referred to as ‘additive light’ and is used in devices that transmit light, such as monitors.

Most LED monitors use a series of Red, Green and Blue pixels to transmit the image we see by mixing varying amounts of RGB. In the most common ‘lower end’ monitors the intensity of each pixel have 256 shades of colour running from black (255) to Full colour (0). The values can be described in a digital from by using 8 bits. Using our 3 primary colours RGB each having 8 bits (24-bit colour) 16.7 million colours can be produced. This is the sRGB Colour Space and it encompasses around 35% of the colours in what is known as the CIE lab colour space.

In 1998 both Adobe and Kodak introduced new colour spaces to cater for the expanding technologies in digital imaging and printing. The Kodak colour space known as Pro Photo RGB is the largest, but 13% of it is outside of the 1931 CIE lab colour space. These colours are therefore ‘imaginary’ and can’t be seen or reproduced; it encompasses 90% of the CIE lab colour Space. There are no devices currently available to view this entire colour space although some high-end printers and monitors can produce some of this colour that falls outside of the Adobe RGB colour space. The Adobe RGB colour space is 50% of the CIE lab colour space. Adobe have adopted the Pro Photo RGB colour space for image editing to ensure that no colour data is lost when editing images from modern digital camera RAW files where 12-14 bit depths are commonly used. It should be noted however, that outputting in Pro Photo RGB is fraught with difficulty and should be avoided, because images printed in this space may not be properly represented

How do we see objects in colour? Objects are seen in colour because they reflect certain wave lengths of light. So a red car absorbs all of the green and blue wavelengths and only reflects the red wavelength. A white car reflects all wavelengths whereas a black car absorbs all wavelengths.

Prints rely on reflected light to allow the image to be viewed. When we print onto a sheet of white paper the absence of ink displays white (the colour of the paper). In the subtractive colour model adding the secondary colours together produces black. However, in practice, Cyan, Magenta, and Yellow do not produce a proper deep black so this is added as an ink (K) in its own right. Printed material therefore uses the CMYK process. A good CMYK printer using the same 8 bit theory as above can usually print the colours within the sRGB colour space, indeed, many high street and online labs only have 4 ink machines and can only accept image files in the sRGB colour space.

In order to enhance the colour fidelity in a print, manufacturers over time have added more inks to their printers, most commonly being light magenta and light cyan. These printers, usually referred to as photo quality printers, easily cover the sRGB colour space and probably some of the Adobe RGB space. However, usually the more inks the better and the bigger the colour space accommodated. The Canon Pro 1000 has 12 inks and can cover all of the Adobe RGB and some of the Pro Photo RGB colour space!  Print quality and fidelity is also affected by the bit depth and therefore it is common practice to output in Adobe RGB 16bit TIFF for printing to get the required colour fidelity.

Colour Management

Is it needed and should I use it – YES and YES

I discussed earlier in this article that the human eye which, when coupled with the human brain is a quite phenomenal piece of imaging equipment. The colour of light that falls on a scene or is reflected from an object is changed by the colour temperature of that light. This is measured in °Kelvin where the lower the temperature the warmer the image. (Why the lower the temperature the warmer the image has always been a subject of confusion and there is a scientific answer but that is not for this missive) It is often said that the colour temperature of ‘daylight’ was determined by Kodak as being noon on a bright sunny day in the state of NY (Kodak HQ). At noon on a sunny day in direct sunlight the light is said to have a ‘colour temperature’ of around 5500°K. In shadows or overcast sunlight the colour temperature increases leading to a bluer or cooler colour.  At sunset or sunrise when the sun being diffused through the atmosphere the colour temperature reduces leading to a warmer or more red/yellow colour. Photographers often refer to this period as the golden hour. Colour temperature in °K is often overlaid on the 1931 CIE diagram to represent this. ‘Day light’ is often described as being either 5500°K (D55) or 6500°K (D65)

Our vision capability is so good that it is often too good! You have all taken a picture indoors using daylight balanced film without a flash! The result when printed is something like the image to the left.

This image was taken in my study with the camera’s white balance set to sunny (to simulate using daylight film). That’s not how it looked to me! My awesome human vision system had already used its ‘auto white balance’ to put the image right! Unfortunately, unlike your camera it is impossible to turn off your own ‘auto white balance’ The human brain will always try and provide you with what it believes as the colour truth.

Whether you look at your monitor or indeed a printed image, what you are seeing is not necessarily the truth: it could be your brain making adjustments that make it look like it is the correct colour You can’t simply look at a monitor and tell that what you are seeing is correct! Likewise, unless a print is viewed under the correct lighting conditions the reflected colours that you ‘see’ could be inaccurate..

Monitor Calibration

There are two main issues with monitors; colour accuracy and brightness. You simply can’t tell by viewing the monitor. An online lab will have their systems colour managed to produce a print with the right density (brightness) and accurate colour.  If your monitor is too bright you will invariably send off a file that has had its brightness reduced to display properly on your screen. When you get the print back it will be too dark. If the colour of your screen is not accurate the print you get back will not match the colour that you had on your screen. Online labs will not usually be responsible for correcting your prints. You may get this service from a high street retailer who should have a system that is colour managed, but you will pay a high premium. Most online labs will warn you that they expect you to have viewed/edited your image on a calibrated monitor.

So how does it work and what do you use?

You will need a screen calibration device known a colourimeter (Calibrite display Pro or DataColor Spyder) plus the relevant software.

The calibration device measures the ambient room brightness and sets a target brightness for the monitor (this is usually well below what is normally set; mine is at around 20% brightness or 120 cd/m2). You decide whether to profile your monitor at D55 or D65.

The software sends known target colour values to your screen and measures the colour. These colours are sent as various values of Red, Green and Blue from black through to white and then various shades of grey from black through to white. The software then produces a ‘correction’ profile using the difference between what your monitor is displaying and the actual target colour generated by the software. This is profile loaded into your computer and used by the GPU in your computer to provide an accurate screen colour calibration.  Because a normal RGB monitor relies on backlighting which over time can fade or change colour, this process should be repeated around once every 2 months to account for the monitors drift in colour. Once done you know that you monitor is giving you true RGB colours.

Room lighting and wall colour can make a difference to monitor colour and. whilst this effect is not drastic, it can be minimised by using a hood to shield the display from ambient light.

Having produced a profile for your monitor, you now have a colour calibrated image. You can now confidently send-off files for printing externally at online labs. However, it would always be prudent to ask for a test print before your first order, before routinely using a lab.

Print Calibration

If you are printing your own images, the aim would be to faithfully reproduce what you see on your ‘profiled’ screen to what you see in your print when viewed under the correct illumination. However, bear in mind that when viewing your monitor, you are looking at a transmitted image and when viewing your print you will be viewing a reflected image. Whilst they should retain the same colour information they will never ‘look’ the same. Even different paper types may look similar bit will never look the same due to the way in which they reflect the light; eg glossy paper will always look richer and more vibrant than matt.

Printer profiling is a little more involved than monitor profiling but is nonetheless easy and essential for getting prints to have the correct colour fidelity.

Profiling is usually done in conjunction with a paper manufacturer such as Fotospeed or Permajet. Although some manufacturers will only supply generic profiles for each of their products. Where possible ‘Custom profiles’ will always have the edge.

Download the profiling pack from the manufacturer’s web site, this will usually include a printer target file, instructions, a profile ordering sheet, and the Adobe Colour Printer Utility.

The International Colour Consortium Custom (ICC) was established in 1993 by eight industry vendors to create, promote, and standardize an open, vendor-neutral, cross-platform colour management systems. Its purpose is to ensure colour consistency across different devices and software. ICC Profiles are specific to an ink type, paper type and printer. So, you should have a separate profile for each of the papers that you prefer to use. These are provided free of charge and you can usually order up to 6 at any one time. Most manufacturers also provide generic profiles for some specific printers (usually the pro models from Epson or Canon).

Once you have the profile pack, print the target patches using the Adobe Colour Printer Utility using the instructions provided. This will ensure that your target is printed WITHOUT any colour management applied by your image processing software, your printer or your computer.

Once printed, send your target print off to the required manufacturer. They will apply your target to a Spectrophotometer that will read the value of each patch. This will produce an ICC profile for your specific printer/ink combination and the selected paper.

After a couple of days you will receive the profile by Email. Follow the instructions and add the profile to your computer.

Now when you print from your profiled monitor to your profiled printer you should get images that match in terms of colour, however, as stated above don’t expect them to look the exactly the same due differing paper types and the fact that monitors and prints are transmissive and reflective. Some manufacturers will provide you with a test print to use with your chosen paper using the provided printer profile to check for accuracy. You will find various test images on the KLDCC web site

All you need to do now is ensure that the prints are viewed under the appropriate lighting conditions either D55 or D65.

Congratulations you now have a colour managed workflow!

Brian Sadler ARPS